Graphs (Calculations Further Down)

piezoelectric testing

Equation Logic for Predicted Sensor Function:

IF (Expected Voltage < 0.019  [ sensor minimum ] )

Expected Voltage = 0.019
ELSE	
	Expected Voltage = (SLOPE * MPH) + (Y-INTERCEPT)
	[ y = mx + b ]

Testing Conditions

| Raw Testing Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| q (in H2O ideal) | q (in H2O DAQ) |  | q (in H2O) | q (Pa) | Fan Spin | V (m/s) |
| -0.008 | -0.008 |  | 0 | 0 | not spinning | 0 |
| 0.001 | 0.001 |  | 0.009 | 2.2392 | not spinning | 1.948 |
| 0.030 | 0.034 |  | 0.042 | 10.4496 | not spinning | 4.208 |
| 0.077 | 0.077 |  | 0.085 | 21.148 | spinning | 5.987 |
| 0.144 | 0.149 |  | 0.157 | 39.0616 |  | 8.137 |
| 0.229 | 0.228 |  | 0.236 | 58.7168 |  | 9.976 |
| 0.333 | 0.337 |  | 0.345 | 85.836 |  | 12.062 |
| 0.457 | 0.458 |  | 0.466 | 115.9408 |  | 14.018 |
| 0.599 | 0.603 |  | 0.611 | 152.0168 |  | 16.052 |

Sensor Best Fit Calculations

| Voltage Parsed | Average Voltage | Veloctity Test Phase |
| --- | --- | --- |
| 0.042 | 0.019 | 1 |
| 0.019 |  |  |
| 0.019 |  |  |
| 0.019 |  |  |
| 0.019 |  |  |
| 0.019 | 0.019 | 2 |
| 0.019 |  |  |
| 0.026 |  |  |
| 0.019 |  |  |
| 0.019 |  |  |

| Line of best fit calculations |  | y-intercept | -0.1687017263 |
| --- | --- | --- | --- |
|  |  | slope | 0.0138 |
| mph_filtered | V_filtered | mph_line | V_line |
|  |  | 0.00 | 0.019 |
|  |  | 4.36 | 0.019 |
|  |  | 9.41 | 0.019 |
| 13.39 | 0.02975 | 13.39 | 0.01607773286 |
| 18.20 | 0.076 | 18.20 | 0.08242564748 |
| 22.32 | 0.1305 | 22.32 | 0.1391915679 |
| 26.98 | 0.19775 | 26.98 | 0.2035646195 |
| 31.36 | 0.2586666667 | 31.36 | 0.2639487509 |
| 35.91 | 0.33925 | 35.91 | 0.326708348 |

Excel formula used to populate Predicted Sensor Function Graph:

=IF((($AC$22 * AH2)+($AC$21))<0.019,0.019,($AC$22 * AH2)+($AC$21))

Pseudo Code Version:
IF (Expected Voltage < 0.019  [ sensor minimum ] )

Expected Voltage = 0.019
ELSE
	Expected Voltage = (SLOPE * MPH) + (Y-INTERCEPT)
	[ y = mx + b ]

An alteration of this formula can be used to know what MPH air is moving at based on Voltage